MATLAB/Simulink Simulation: Closed-Loop Buck Converter

This guide provides a step-by-step walkthrough for creating a closed-loop buck converter simulation. The goal is to regulate the output voltage to a desired setpoint, even when the input voltage or load changes.

Part 1: Understanding the Circuit and Control

A buck converter is a DC-to-DC power converter that steps down voltage. A closed-loop system uses feedback to maintain a constant output.

- Power Circuit: Consists of an input voltage source, a switching element (MOSFET), a diode, an inductor (L), a capacitor (C), and the load (R).

- Control Circuit:

  - Measures the output voltage (V_out).

  - Compares V_out to a desired reference voltage (V_ref) to find the error.

  - A PI (Proportional-Integral) Controller processes this error and calculates a new duty cycle.

  - A PWM (Pulse Width Modulation) Generator creates the switching signal for the MOSFET based on the PI controller's output.

Part 2: Step-by-Step Simulink Model Construction

Open MATLAB and then open Simulink. Start a new, blank model (Ctrl+N). We will use blocks from the Simscape Electrical library.

Step 1: Build the Power Circuit

First, we'll lay out the main components of the buck converter.

- Add Blocks: From the Simulink Library Browser, find and drag the following blocks into your model.

  - DC Voltage Source

  - MOSFET

  - Diode

  - Series RLC Branch (We will use three of these: one for the inductor, one for the capacitor, and one for the load resistor).

  - Voltage Measurement

  - Ground (from Simscape > Electrical > Specialized Power Systems > Passives)

  - powergui (This block is essential for any Simscape Electrical model).

- Configure and Connect the Blocks:

  - DC Voltage Source:

    - Connect its positive (+) terminal to the drain of the MOSFET.

    - Connect its negative (-) terminal to the Ground.

  - MOSFET:

    - The drain is already connected.

    - Connect the source to the anode (+) of the Diode and one end of the inductor Series RLC Branch.

  - Diode:

    - The anode (+) is already connected.

    - Connect the cathode (-) to the Ground.

  - Inductor (Series RLC Branch 1):

    - In the block parameters, set Branch Type to L.

    - Connect it between the MOSFET source and the output node.

  - Capacitor (Series RLC Branch 2):

    - In the block parameters, set Branch Type to C.

    - Connect it in parallel with the load resistor. One end goes to the output node (after the inductor), and the other end goes to Ground.

  - Load (Series RLC Branch 3):

    - In the block parameters, set Branch Type to R.

    - Connect it in parallel with the capacitor.

  - Voltage Measurement:

    - Connect the + and - terminals across the load resistor to measure V_out.

Your power circuit should look like this:

Step 2: Build the Control Circuit

Now, we create the feedback loop that regulates the voltage.

- Add Blocks: Find and drag these standard Simulink blocks.

  - Constant (for the reference voltage)

  - Sum

  - PID Controller

  - Repeating Sequence (to create the PWM carrier wave)

  - Relational Operator

- Configure and Connect the Blocks:

  - Constant (V_ref): This is your desired output voltage.

  - Sum:

    - Connect the Constant block to the + input.

    - Connect the output of the Voltage Measurement block (V_out) to the - input.

    - The output of this block is the error signal.

  - PID Controller:

    - Connect the error signal from the Sum block to the input of the PID Controller.

    - In the block parameters, select the controller type PI. You can leave the derivative gain D as 0.

  - Repeating Sequence (Sawtooth Wave):

    - This block will generate a triangular/sawtooth wave, which is the carrier for our PWM signal.

    - Set Time values to [0 1/fsw] where fsw is your switching frequency (e.g., 1/25000).

    - Set Output values to [0 1]. This creates a wave that ramps from 0 to 1.

  - Relational Operator:

    - Set the operator to >.

    - Connect the output of the PID Controller to the top > input.

    - Connect the output of the Repeating Sequence block to the bottom input.

    - The output of this block is the PWM signal. It will be 1 when the PI output is greater than the sawtooth wave and 0 otherwise.

- Connect Control to Power:

  - Take the output signal from the Relational Operator and connect it to the gate (G) port of the MOSFET.

Your complete model should now look like this:

Step 3: Add Scopes and Run

- Add a Scope block.

- Use a Mux block to view multiple signals on the same scope. Connect the Constant (V_ref) and the Voltage Measurement (V_out) to the scope to compare them.

- Double-click the powergui block and ensure the Simulation type is set to Continuous.

- Set your simulation stop time (e.g., 0.1 seconds).

- Press the Run button.

Part 3: Block Parameters Summary

Here are the recommended starting parameters for each block.

| Block Name | Parameter | Value |
| --- | --- | --- |
| Power Circuit |  |  |
| DC Voltage Source | Amplitude (V) | 24 |
| MOSFET | Internal diode | Unchecked (we use an external one) |
|  | Resistance Ron (Ohms) | 0.01 |
| Diode | Forward Voltage (V) | 0.7 |
|  | Ron (Ohms) | 0.001 |
| Series RLC Branch (L) | Branch Type | L |
|  | Inductance (H) | 1e-3 (1 mH) |
| Series RLC Branch (C) | Branch Type | C |
|  | Capacitance (F) | 470e-6 (470 µF) |
| Series RLC Branch (R) | Branch Type | R |
|  | Resistance (Ohms) | 10 |
| Control Circuit |  |  |
| Constant (V_ref) | Constant value | 12 |
| PID Controller | Controller | PI |
|  | Proportional (P) | 0.45 (Tune this value) |
|  | Integral (I) | 150 (Tune this value) |
| Repeating Sequence | Time values | [0 1/25000] |
|  | Output values | [0 1] |
| Relational Operator | Operator | > |
| Simulation |  |  |
| powergui | Simulation type | Continuous |
| Solver | Stop time | 0.1 |
|  | Solver | auto (ode23tb) |

Expected Results

When you run the simulation, the scope should show the output voltage starting from 0, rising, slightly overshooting the 12V reference, and then settling exactly at 12V. The PI controller continuously adjusts the PWM duty cycle to keep it there.